# Project: Parametric 3D-Printed Tapered Tenon Templates for the Woodpeckers Multi-Router

## Goal

I own a Woodpeckers Multi-Router and currently use the standard Woodpeckers tenon templates.

I want to develop a simple application that allows me to enter parameters such as:

- Router bit diameter

- Tenon width

- Tenon thickness

- Tenon length

- Desired taper / fit adjustment

and automatically generates an STL file for a physical template that fits directly into the existing Woodpeckers Multi-Router template holder.

The key feature I want to borrow from the PantoRouter is the use of a tapered template profile, which allows the user to progressively refine the tenon fit by moving the Multi-Router stylus along the taper.

The ultimate goal is therefore:

A parametric Multi-Router template generator that produces Woodpeckers-compatible templates with PantoRouter-style tapered tenon profiles.

# Existing Equipment

The machine is a Woodpeckers Multi-Router.

I currently own and use the Woodpeckers factory tenon templates and template plate/holder.

The desired printed templates must physically fit into the existing Woodpeckers template interface. I do NOT want to modify the Multi-Router itself.

The concept is to reproduce the physical interface of the Woodpeckers template while changing the actual template profile to provide adjustable taper.

# Relevant Woodpeckers Multi-Router Concept

The Multi-Router uses a stylus/bearing that follows the perimeter of a template while the router bit cuts the corresponding tenon.

The template therefore determines the resulting tenon geometry through the dimensional relationship between:

- The template profile

- The stylus/bearing

- The router-bit diameter

- The Multi-Router's mechanical geometry

The Woodpeckers system uses dedicated tenon templates that mount into a template holder/plate.

The exact geometry of that mounting/interface needs to be measured or reverse-engineered rather than guessed.

The existing Woodpeckers templates should be treated as the dimensional reference for:

- Template thickness

- Overall template dimensions

- Mounting/locating features

- Template-holder engagement

- Centerline/reference geometry

- Any stops or other features needed for proper positioning

# Relevant PantoRouter Concept

The PantoRouter uses tapered templates rather than simply having one fixed template for each tenon dimension.

The tapered template allows the operator to move the stylus along different portions of the taper to make progressively different tenon dimensions.

This makes it possible to sneak up on a precise furniture-joint fit without having to manufacture a new fixed template for every slight dimensional adjustment.

This is the behavior I want to reproduce on the Multi-Router.

I do NOT necessarily want to copy the PantoRouter's physical template dimensions or mounting system.

I want to copy the functional concept:

A controlled taper in the template profile produces a controlled change in the resulting tenon dimension as the stylus position is changed.

The taper therefore needs to be mathematically derived for the Multi-Router's geometry rather than blindly copying the apparent angle of a PantoRouter template.

# Desired Application

I want a very simple application.

Conceptually, the UI might look something like:

## Tenon Parameters

Router bit diameter:
[ 0.500" ]

Tenon width:
[ 1.000" ]

Tenon thickness:
[ 0.500" ]

Tenon length:
[ 2.000" ]

Taper / adjustment range:
[ 0.020" ]

Taper direction:
[ ... ]

Then the application generates a template and provides:

[Generate STL]

The STL should be directly printable on a normal FDM 3D printer.

The application should ideally also generate useful reference marks on the template corresponding to known dimensional adjustments.

For example:

- 0.000"

- -0.005"

- -0.010"

- -0.015"

- -0.020"

or whatever adjustment range makes sense for the actual Multi-Router geometry.

The exact relationship between stylus position and resulting tenon dimension must be calculated rather than assumed.

# Important Geometric Requirement

The application cannot simply draw a tapered rectangle and export it.

The template profile must account for the actual Multi-Router geometry.

The critical relationship is:

template surface → stylus bearing → router-bit centerline → resulting tenon surface

The application should therefore establish a mathematical transformation from desired tenon geometry to the required template geometry.

The user's router bit diameter should be an input parameter because different bit diameters will change the required template geometry.

The stylus/bearing diameter is also likely important and may need to be either:

- A fixed machine-specific value, if the Multi-Router stylus is standardized, or

- A user-configurable parameter.

The application should make this explicit rather than hiding an assumed value.

# Desired Template Design

The printed template should ideally consist of a standard Woodpeckers-compatible mounting/interface geometry plus a parametric profile.

Conceptually:

Woodpeckers mounting/interface

┌──────────────────────────────┐

│                              │

│      tapered guide profile   │

│       \                 /    │

│        \               /     │

│         \_____________/      │

│                              │

└──────────────────────────────┘

The exact shape above is only conceptual.

The actual profile needs to be derived from the Multi-Router's stylus and router-bit geometry.

The printed template should be sufficiently thick and rigid for the stylus bearing to follow its edge repeatedly.

# Potential Template Strategy

One particularly useful implementation would be a single continuously tapered template rather than making separate templates for every possible tenon fit.

For example, the template could represent a range such as:

- Nominal

- -0.005"

- -0.010"

- -0.015"

- -0.020"

The template could have engraved/embossed reference markings indicating the approximate resulting tenon size at each location.

This would allow the user to:

- Cut a test tenon.

- Check the fit.

- Move the stylus to another position.

- Cut another tenon.

- Continue until the desired fit is achieved.

This is the primary reason for wanting the PantoRouter-style taper.

# Important Design Question: What Exactly Does the Taper Control?

We need to determine whether the desired taper should control:

- Tenon thickness only

- Tenon width only

- Both simultaneously

- A user-selectable combination

For a typical mortise-and-tenon application, the most useful behavior may be a taper that changes the tenon thickness while keeping the tenon length and width constant.

However, this needs to be established from the PantoRouter's actual geometry and the Multi-Router's kinematics.

# Potential Future Features

Once the underlying geometry is correct, the application could eventually support:

### Basic

- Bit diameter

- Tenon thickness

- Tenon width

- Tenon length

- Taper amount

- Adjustment increment

- STL export

### More advanced

- Mortise dimensions

- Tenon shoulder dimensions

- Multiple taper profiles

- Reference markings

- Multiple tenons on one template

- Angled tenons

- Offset tenons

- Asymmetric tenons

- Custom furniture-joint geometry

The initial version should remain simple and focus on generating a correct standard tapered tenon template.

# 3D Printer Requirements

The templates do not require extreme 3D-printing resolution.

The important characteristics are:

- Dimensional repeatability

- XY accuracy

- Flatness

- Consistent extrusion

- Adequate stiffness

- Wear resistance against the stylus bearing

The initial printer recommendation was a Bambu Lab P1S, largely because it offers good repeatability and automation without requiring an unnecessarily expensive industrial printer.

A Prusa CORE One+ was also considered and may be preferable if dimensional accuracy/repeatability and open ecosystem/calibration are prioritized over maximum automation.

The exact printer is secondary to getting the template geometry correct.

# Suggested Printing Parameters

These are preliminary recommendations, not established requirements:

- FDM printing

- 0.4 mm nozzle

- Approximately 0.10–0.16 mm layer height

- 5–6 perimeter walls

- High infill, potentially 100%

- PETG as an initial material

PLA could potentially be dimensionally excellent but may be less desirable for a template whose edge is repeatedly contacted by a bearing.

PETG or another tougher engineering plastic may provide better wear resistance.

A future version could potentially use a replaceable wear strip or harder material along the template's bearing-contact surface.

# Critical Open Questions / Measurements

Before writing the actual geometry generator, these need to be established.

## 1. Exact Multi-Router model/version

Determine exactly which Woodpeckers Multi-Router and template system I have.

Questions:

- Original Multi-Router or current version?

- Original tenon template system or newer MT system?

- Exact template holder/plate design?

Photos of the machine, template, and template holder would be useful.

## 2. Existing template dimensions

Measure an original Woodpeckers tenon template.

Important measurements:

- Overall length

- Overall width

- Thickness

- Location and dimensions of mounting features

- Location of locating features

- Centerline

- Template profile dimensions

- Any slots, holes, or stops

- Distance between relevant reference surfaces

The factory template should be used as the dimensional reference for the printed template interface.

## 3. Stylus/bearing diameter

Determine the exact diameter of the Multi-Router stylus bearing.

This is extremely important because the template-to-bit offset depends on it.

Measure with calipers or preferably a micrometer.

Also determine:

- Bearing width

- Whether the bearing is fixed or replaceable

- Exact point at which the bearing contacts the template

## 4. Router-bit diameter

Determine the actual router bit used for the tenon operation.

The application should ideally allow the user to enter bit diameter.

At minimum, determine the standard 1/2" bit geometry used by the existing Woodpeckers templates.

Actual measured diameter may be more useful than nominal diameter because router bits can vary slightly.

## 5. Stylus-to-router relationship

Determine the exact mechanical relationship between the stylus and router bit.

Specifically:

- Distance between stylus centerline and router-bit centerline

- Direction of movement

- Whether this distance changes with different bit configurations

- Whether the stylus and router move together in a simple fixed offset

This is required to derive the template profile mathematically.

## 6. Existing Woodpeckers template geometry

Measure a factory template for a known tenon.

For example:

- Nominal 1/2" tenon

- Known template dimensions

- Known bit diameter

- Known stylus diameter

Then compare the resulting physical tenon to the theoretical geometry.

This provides a real-world calibration point for the mathematical model.

## 7. Direction of template taper

Determine exactly how the PantoRouter taper functions.

We should inspect the PantoRouter documentation/product page and, if possible, actual template geometry to determine:

- Which template surface is tapered

- Which dimension changes as the stylus moves

- Whether both sides taper

- Whether the taper is linear

- How much dimensional change corresponds to a given distance along the template

- Whether the PantoRouter template is intentionally oversized/undersized relative to the finished tenon

Do not assume the visible taper angle directly corresponds to tenon taper.

## 8. Desired adjustment range

Determine a useful default range.

A starting hypothesis might be:

0.000" to 0.020" adjustment

with markings every:

0.005"

But this should be validated against actual furniture-joint fitting requirements.

It may be more useful to specify the adjustment as:

total tenon thickness change

rather than:

taper angle

The user should care about the resulting joint fit, not the physical taper angle.

# Recommended Development Process

## Phase 1: Reverse-engineer the factory template

Take an existing Woodpeckers template and document:

- Dimensions

- Mounting geometry

- Stylus relationship

- Router-bit relationship

- Resulting tenon geometry

Create a CAD model of the factory template.

## Phase 2: Validate the geometric model

Use the known factory template to calculate the expected tenon.

Compare:

CAD prediction vs. actual factory template vs. actual cut tenon.

Adjust the mathematical model until it accurately reproduces the factory template.

This is important because a purely theoretical model could overlook mechanical offsets in the Multi-Router.

## Phase 3: Add PantoRouter-style taper

Once the factory template is understood, modify only the profile.

Create a continuously tapered profile where stylus position corresponds to a known tenon dimensional adjustment.

For example:

Stylus position

↓

0.000" ─────────── nominal

0.005" ─────────── slightly loose

0.010" ─────────── looser

0.015" ─────────── looser

0.020" ─────────── looser

The exact direction should be determined experimentally.

## Phase 4: Print a calibration template

Print a test template with exaggerated reference markings.

Cut several tenons.

Measure:

- Tenon thickness

- Tenon width

- Position-dependent variation

- Actual taper

Compare actual measurements against predicted values.

Use these results to calibrate the generator.

## Phase 5: Build the simple application

The application should ultimately let the user enter:

Bit diameter:       0.500"

Tenon width:        1.000"

Tenon thickness:    0.500"

Tenon length:       2.000"

Adjustment range:   0.020"

and produce:

multirouter_tenon_template.stl

Potentially also produce a small text report containing:

- Nominal tenon dimensions

- Adjustment range

- Reference-mark dimensions

- Printer recommendations

- Suggested print orientation

# Most Important Principle

Do not begin by trying to make the STL.

First determine the mathematical relationship between the Woodpeckers Multi-Router's stylus, template, router bit, and resulting tenon.

Once that relationship is known, the STL generation should be relatively straightforward.

The ultimate objective is not merely:

"Make a tapered piece of plastic that fits the Multi-Router."

It is:

"Given a desired tenon geometry and router-bit diameter, calculate the exact Woodpeckers-compatible template profile that causes the Multi-Router stylus to produce that geometry, while incorporating a controlled PantoRouter-style taper for fit adjustment."

That is the core engineering problem.